Posterior Ankle Arthoscopy – Mr Gordon (NHS)

Antibiotics:	None

Tourniquet:	Thigh, Kimberly – Clarke blue wrap, sealed with Leukoplast tape, sterile 6 inch Esmarch, 250mmHg once draped

Traction:	None

Position:	Prone, cylindrical bolster under shin with sandbag under bolster, calf pumps, curved thigh gutter support

Skin Prep:	Supine - Alcoholic Chlorhexadine in Mr Gordon’s Sterile Isolation Bag (for foot).  Turn prone, then with swabs to above knee

Drape:		Trolley cover, Extremity Drape

Block:		Popliteal (normally done by anaesthetist) – if not: 20mls 0.5% Bupivacaine

Equipment:	Knee scope (4.2mm)

Knee shaver with pedals

Knee shaver blade: 4.8mm Cuda (Conmed)

1 x 3L cold saline, free drainage (no pump), set at maximum drip stand height

Arthroscopy irrigation set

Sterile 6 inch Esmarch

20 mls syringe filled with irrigation fluid + white needle

1 x #11 blade 	Straight mosquito 	Arthroscopic probe

Diathermy bag and quiver

On Standby:	Shaver blades: Cuda (Conmed): 3.5mm

Ankle arthroscopic instruments (if ordered)

Arthrowand Super Turbo Vac 90 (Arthrocare) + second suction tubing

Sutures:		3/0 Ethilon

Dressings:	Opsite, gauze (2 to each wound, folded into a square)

Wool - 1 x 4 inch, Crepe - 1 x 4 inch

Rigid flat sandal

Post op:		Home same day

FWB

Strict elevation 1/52, 23 hours/day

Bandage off by patient 48 hours and leave Opsite

Foot, ankle, knee active ROM

2/52 – OPD: wound check