Subtalar Fusion Arthoscopic – Mr Gordon (Harpenden)

Antibiotics:	None

Tourniquet:	Thigh, Kimberly – Clarke blue wrap, sealed with Leukoplast tape, sterile 6 inch Esmarch, 250mmHg once draped

Traction:	None

Position:	Prone, cylindrical bolster under shin with sandbag under bolster, calf pumps, curved thigh gutter support,

Skin Prep:	Alcoholic Chlorhexadine in Mr Gordon’s Sterile Isolation Bag (Vi-Drape, MCD, ref: D37105510 (for foot) then with swabs to above knee

Drape:		Trolley cover, Extremity Drape

Block:		20mls 0.5% Bupivacaine at end

Equipment:	Knee scope (4.2mm)

Knee shaver with pedals

Knee shaver blade: 5.5mm Bone Cutter (Smith & Nephew), 4.0mm Acromioblaster burr

Ankle arthroscopic instruments (Smith & Nephew) – needs to be ordered

Fine osteotomes

6.5mm cannulated screw set (inc Threaded guide, pin Cannulated drill)

Large Drill (Battery)

1 x 3L cold saline, free drainage (no pump), set at maximum drip stand height

Single chamber giving set

Sterile 6 inch Esmarch

1 x #11 blade

Straight mosquito

Arthroscopic probe

On Standby:	Shaver blades: 4.5mm Bone Cutter (Smith & Nephew)

Arthrowand Super Turbo Vac 90 (Arthrocare) + second suction tubing

Sutures:		3/0 Ethilon

Dressings:	Opsite

6” Velband / 6” Crepe bandage

Below Knee plaster back slab

Post op:		Home next day

TWB

Strict elevation 2/52, 23 hours/day

2/52 – OPD: wound check