1st MTPJ (Big Toe Joint) Arthroscopy – Mr Gordon (Bushey)

Antibiotics:	None

Tourniquet:	Velban, arm tourniquet, Leukoplast tape

Inflated to 250mmHg after sterile 4 inch Esmarch applied by surgeon

Position:	Supine, calf pump, sandbag under ipsilateral buttock

Skin Prep:	Alcoholic Chlorhexadine in Mr Gordon’s Sterile Bag

Drape:		Trolley cover, Hand & Foot drape, Clipped

Ankle Block:	20mls 0.5% Bupivacaine (normally done by anaesthetist)

Equipment:	2.0mm arthroscope

Small shaver with pedals

Shaver blades - 2.0mm Cuda (Conmed)

Ankle microfracture instruments, Arthrex

1 x 3L cold saline, free drainage (no pump), set at maximum drip stand height

Arthroscopy irrigation set

Or

Pump set at 30mmHg

4 inch sterile Esmarch

1 x #15 Blades

Sutures:		2/0 Prolene (8026T), ½ “ steristrips cut in half

Dressings:	2 x Blue Gauze

4” Velband

4” Crepe bandage

Postop Rehab:	Walking boot applied on discharge

Full weight bearing in boot

Home same day

F/up at 2 (wound review) + 6 wks

Physio at 6/52 if necessary