Subtalar Fusion Arthoscopic – Mr Gordon

Antibiotics:	Yes

Tourniquet:	Thigh, Kimberly – Clarke blue wrap, sealed with Leukoplast tape, sterile 6 inch Esmarch, 250mmHg once draped

Traction:	None

Position:	Recovery position, calf pumps

Lateral with supports (as for hip replacement)

Right angled bolster under shin

Skin Prep:	Supine - Alcoholic Chlorhexadine in Mr Gordon’s Sterile Isolation Bag (for foot).  Turn patient, then with swabs to knee

Drape:		Trolley cover, shoulder arthroscopy drape

Block:		20mls 0.5% Bupivacaine at end

Equipment:	Knee scope (4.2mm)

Knee shaver with pedals

Knee shaver blade: 4.8mm Cuda (Conmed) / 4.5mm Incisor Plus (Smith & Nephew)

Ankle arthroscopic instruments

Mercier fine osteotomes

Orthosolutions 6.5mm cannulated screws set

-Cannulated drill 5x150mm (2461-14-629)

-Threaded guide pin x 2 (2461-40-129)

Stryker Large Drill

1 x 3L cold saline, free drainage (no pump), set at maximum drip stand height

Single chamber giving set

Sterile 6 inch Esmarch

1 x #11 blade 	Straight mosquito 	Arthroscopic probe

On Standby:	Shaver blades: Cuda (Conmed) / Incisor Plus (Smith & Nephew): 3.5mm, 2.9mm, 2.0mm

Ankle scope (2.7mm)

Arthrowand Super Turbo Vac 90 (Arthrocare) + second suction tubing

Sutures:		3/0 Ethilon and No.1 vicryl ties – 2 x 5 cm cut pieces

Dressings:	Opsite

6” Velband / 6” Crepe bandage

Below Knee plaster back slab

Post op:		Home same day / next day

TWB

Strict elevation 1/52, 23 hours/day

2/52 – OPD: wound check