1st MTPJ (Big Toe Joint) Arthroscopy – Mr Gordon (NHS)

Antibiotics:	IV on induction

Tourniquet:	Sterile 4 inch Esmarch applied by surgeon

Position:	Supine, calf pump, sandbag under ipsilateral buttock

Skin Prep:	Alcoholic Chlorhexadine in Mr Gordon’s Sterile Bag

Drape:		Trolley cover, Hand & Foot drape, clipped with Kelly clamp

Ankle Block:	20mls 0.5% Bupivacaine (normally done by anaesthetist)

Equipment:	2.0mm arthroscope

Small shaver with pedals

Shaver blades - 2.0mm Cuda (Conmed)

Ankle microfracture instruments

1 x 3L cold saline, free drainage (no pump), set at maximum drip stand height

Arthroscopy irrigation set

4 inch sterile Esmarch

1 x #15 Blades

Sutures:		½ “ steristrips cut in half

Dressings:	2 x Blue Gauze

4” Velband

4” Crepe bandage

Postop Rehab:	Flay post op sandal or walking boot applied on discharge

Full weight bearing

Home same day

F/up at 2 (wound review) + 6 wks

Physio at 6/52 if necessary