Minimally Invasive Chevron Akin (MICA) Hallux Valgus Surgery - Mr Gordon

Antibiotics:	IV on induction

X-Ray:	Mini C-Arm with cover (or image intensifier if unavailable)

Position:	Supine, calf pump (no sandbag)

Skin Prep:	Alcoholic Chlorhexadine in Mr Gordon’s Sterile Bag

Drape:		Medium (ground sheet), Hand & Foot drape

Tourniquet:	4 inch Esmarch bandage, clipped with Kelly

Equipment:	Wright Medical Burr Machine with sterile hand piece, irrigation tubing and saline bag

Small electric powered drill with quick couple attachments

eg. Conmed Linvatec Drill (Handpiece Pro 6100), Foot De Soutter, Stryker

Wright Medical Dart-Fire cannulated screw set

Implants and Consumables: Wright Medical (Instruments and screws), Stryker Fixos 4mm screws

Open at start of case:

Burrs: 		Shannon burr 2x20mm (for Chevron)

K-wires: 		2 x 2mmx200mm single ended K wire

Disposables:	MI Instrument Set, MICA screw driver, depth guage

4 inch sterile Esmarch, Beaver blade (SM 64)

Saline wash with 2 x 20 ml syringes and 14G (orange) Abbocath arterial catheter

Screw Set: 	screw set

Open once advised during case:

Screws: 		MI 3.0mm (32-44mm) screws (length once measured)

Stryker Fixos 4mm screw

Wright Medical HV (10-40mm) (for Akin)

Burrs: 		Shannon burr 2mm x 12mm (for Akin)

Wedge burr 3.1mm x 13mm (for bunionectomy/spikectomy)

Drills:		1 x 2.0mm cannulated ‘tube’ drill (if Akin)

Sutures:		½“ (blue) steristrips cut in half

Dressings: 	Jelonet 10x10cm

1 x Dressing gauze pack (5 pieces)

Peha-haft 10cm wide

Rigid flat black sandal (label inside at heel, M=male, F=female, Small/Medium/Large)

Post op Rehab:	Rigid flat sandal 6 wks, Home same day if safe, FWB, F/up at 2 (wound review) + 6 wks (Xrays on 		arrival), Physio at 6/52 if necessary